# Written Report

Cole Geer, Marissa Faue

Project #2: Automation of Concrete Mix Design

Date Submitted for Review: 02/24/2026

Client: Nebraska Department of Transportation (NDOT)

## Introduction / Background

The Nebraska Department of Transportation (NDOT) relies on a mix design worksheet to calculate material quantities needed to produce concrete that meets specification requirements. These calculations are often performed in Excel, which can make it difficult to track the logic behind each step or quickly compare different mix scenarios (Nebraska Department of Transportation, n.d.).

The goal of this project was to automate the NDOT mix design worksheet in Python by converting each key spreadsheet calculation into a function-based workflow. The program collects user inputs in a controlled sequence, computes intermediate values needed for volume, and outputs a final weight chart for one cubic yard of concrete. To verify the model works correctly, four different mix scenarios were tested and compared in accordance with requirements outlined in the NDOT client prompt (Nebraska Department of Transportation, n.d.).

## Methods

This project was completed using Python in a Jupyter Notebook environment by turning the Nebraska Department of Transportation (NDOT) concrete mix design worksheet into a structured and function-based workflow. User inputs were collected using Python’s input() function to represent the order of the Excel worksheet and reduce the chance of input errors. Inputs included cementitious material weights, target water–cement ratio, air content, aggregate proportions, and specific gravities for all materials. These inputs were then passed into individual Python functions that replicate the spreadsheet calculations, including computation of water weight, conversion of cementitious materials to volumes, calculation of air and water volumes, and determination of total available aggregate volume for one cubic yard of concrete. Aggregate weights for fine, coarse, and other aggregates were calculated based on target percentages and material specific gravities. The workflow concludes by printing a formatted weight summary for one cubic yard of concrete.

The required input parameters for the automated model include cementitious material weights, water–cement ratio, air content, aggregate proportions, and material specific gravities. The primary output parameters are the calculated material weights for one cubic yard of concrete, including water weight, fine aggregate weight, coarse aggregate weight, and other aggregate weight, along with intermediate volume calculations used to complete the design. Because each calculation step is executed in a consistent order using defined functions, the analysis is fully replicable, and another user can obtain identical results by entering the same input values.

## Results and Discussion

The automated workflow successfully produced a complete weight summary output for each scenario tested. All scenarios produced reasonable differences in material quantities based on changes in cementitious content, water–cement ratio, air content, and aggregate proportions.

#### Scenario Results (Final Weight Summaries)

Scenario 1 (Class 47B)

- Cement = 560 lb, Fly Ash = 80 lb, w/c = 0.44, Air = 6%

- Fine Aggregate (Y) = 1201 lb

- Coarse Aggregate (Z) = 1677 lb

- Water (Q) = 282 lb

- Other Aggregate (AA) = 0 lb

This mix represents a standard paving concrete with moderate cementitious content and air suitable for general roadway construction.

Scenario 2 (Class 47BR)

- Cement = 650 lb, Fly Ash = 100 lb, w/c = 0.40, Air = 6.5%

- Fine Aggregate (Y) = 1077 lb

- Coarse Aggregate (Z) = 1633 lb

- Water (Q) = 300 lb

- Other Aggregate (AA) = 0 lb

This mix reflects higher cementitious content and a lower water–cement ratio, making it appropriate for applications requiring increased strength and durability.

Scenario 3 (Class 47B)

- Cement = 600 lb, Fly Ash = 60 lb, Silica Fume = 25 lb, w/c = 0.38, Air = 6%

- Fine Aggregate (Y) = 1264 lb

- Coarse Aggregate (Z) = 1627 lb

- Water (Q) = 260 lb

- Other Aggregate (AA) = 0 lb

This mix incorporates supplementary cementitious material to improve strength and durability and may be used where enhanced performance is required.

Scenario 4 (Class 47BR)

- Cement = 575 lb, Fly Ash = 75 lb, w/c = 0.46, Air = 7%

- Fine Aggregate (Y) = 1326 lb

- Coarse Aggregate (Z) = 1453 lb

- Water (Q) = 299 lb

- Other Aggregate (AA) = 0 lb

This mix represents a design with higher air content and adjusted aggregate proportions, which may be suitable for applications where freeze resistance is a priority.

Across the four scenarios, changes in water–cement ratio and cementitious content were reflected in the water weight (Q), which ranged from 260 lb (Scenario 3) to 300 lb (Scenario 2). Aggregate weights also shifted based on the target fine/coarse percentages and total aggregate volume available after accounting for cementitious, air, and water volumes. Scenarios with higher target fine aggregate percentages produced higher fine aggregate weights, while scenarios with higher air content slightly reduced available aggregate volume and affected the aggregate weights. Overall, the outputs appear logical as inputs changed, which supports that the spreadsheet was successfully translated into a consistent Python workflow that meets NDOT mix design requirements (Nebraska Department of Transportation, n.d.).

## References

Nebraska Department of Transportation. (n.d.). Concrete mix design submittal spreadsheet [Microsoft Excel file]

Nebraska Department of Transportation. (n.d.). Project 2 client prompt: Automation of concrete mix design [Course assignment document]