Project 2 Annotated Code Document

| Code | Annotation |
| --- | --- |
| For Task 1: Calculate Water Weight |  |
| def water_weight(A, B, C, D, E): total_cementitious = A + B + C + D water_weight = total_cementitious * E return water_weight | This function calculates water weight (Q). It adds all cementitious materials together and multiplies by the water–cement ratio. This follows the NDOT formula: Q = (w/c) × total cementitious weight. |
| For Task 2: Convert Cementitious Materials to Volume |  |
| def calculate_volume_R(A, J): return A / (J * 62.4) | This converts cement weight into volume using the absolute volume method: Volume = Weight ÷ (Specific Gravity × 62.4). The same logic is used for fly ash, silica fume, and other SCM materials. |
| For Task 3: Calculate Air and Water Volume |  |
| def calculate_volume_V(F): return (F/100) * 27 | This calculates air volume (V). It converts air percentage into cubic feet based on 27 ft³ per cubic yard. |
| def calculate_volume_W(Q): return Q / 62.4 | This calculates water volume (W). It divides water weight by 62.4 to convert weight into volume. |
| For Task 4: Calculate Total Aggregate Volume |  |
| def calculate_total_agg_volume(R, S, T, U, V, W): return 27 - (R + S + T + U + V + W) | This calculates total aggregate volume (X). It subtracts cementitious, air, and water volumes from 27 ft³ so the total equals one cubic yard. |
| For Task 5: Calculate Aggregate Weights |  |
| def calculate_fineagg_weight(G, N, X): volume_fine = (G/100) * X return volume_fine * N * 62.4 | This calculates fine aggregate weight (Y). It splits total aggregate volume by percentage and converts volume to weight using specific gravity. |
| For Task 6: Run and Compare Mix Scenarios |  |
| def run scenario(name, A, B, C, D, E, F, G, H, I, J, K, L, M, N, O, P): Q = water_weight(A, B, C, D, E) | This function runs the full mix design workflow. It calculates water weight, volumes, aggregate weights, and prints the final weight chart. It was used to test four NDOT mix scenarios. |